Les variables publiques sur les classes sont conseillées par soucis de simplification lorsqu’elles sont des variables de configuration dont les modifications ne déclenchent pas d’action.

Risques :

-Dépassement de stack : ??

Quand un composant a besoin d’un composant du layer d’en dessous, il utilise le « Layer.h » associé. Les composants d’un level ne doivent pas inclure le « Layer.h » de leur level.

Setup gather all global vars so that thread safety problems are easier to catch (as they are always related to a global var)

Always pass via ArdOs to create OS objects so that statistic are possible

Dynamic memory allocation (new/malloc/free/delete) are forbidden at runtime to prevent :

- thread safety  in memory allocation (usually not thread safe in light os)

- memory fragmentation

New are allowed at setup but discouraged (prefer a static assembly). Delete and free are not required as the CPU is always reset after use.

# Tooling

## Atmel

In order to add modules when a project is created with ASF, you need to go to “ASF->ASF Wizard”.

In case it’s needed, the CPU reference is ATSAM3X8E.

Here is a simple main file to blink the amber led “L” (orange) :

#include <asf.h>

int main (void)

{

sysclk_init();

board_init();

gpio_configure_pin(LED0_GPIO, LED0_FLAGS);

gpio_set_pin_low(LED0_GPIO);

while(1)

{

gpio_toggle_pin(LED0_GPIO);

delay_ms(500);

}

}

## Atmel Prog without JTAG

In order to program the Arduino due from Atmel witout JTAG , we use “bossac”. In “Tools->External tools…” :

Add the following “bossac.bat” file in your project :

@mode com3:1200,n,8,1

@sleep 1

@C:\Users\wix\AppData\Local\Arduino15\packages\arduino\tools\bossac\1.6.1-arduino/bossac.exe -i -U false  --port=%1 -e -w -b %2 -R

Note : normally we should be able to call bossac directly from the dev env, but for a strange reason, the serial reconfiguration is used to provoke an hard-reset on the board which is necessary.

Note2: If you can configure Atmel for this, you can also configure an Eclipse. The difference is that Atmel provide facilities to compile as the dev env is integrated.

## Atmel with JTAG:

### Jtag setup:

Connect the board with the following wiring (plug the due-jtag first, plug the jtag usb wire to your PC, then power the board):

It’s possible to test the device connectivity with “Tools->Device programming”, configure the Jtag connection (Tool/Device/Interface) and clic on apply, then go to the “Device information” tab, and clic read.

You should see this :

### Programming an existing binary

In order to program an existing binary, got to the “Memories” tab, select your file and clic on program:

## Sbrk missing

By default no syscall is possible, so if you include a standard library it’ll miss the _sbrk function that is responsible for allocating the heap so that malloc and co can work. In order to instanciate a fake sbrt add the following option in compiler miscellaneous text field :

--specs=nosys.specs